Date: 1 Mar 11

Part No.: PT80273

## CU/MICA/LSOH/OS/FRPVC

## VERMIN-PROOF, UV RESISTING & FIRE RESISTING

## TELEPHONE CABLE

Application:

This cable is used for communication and telephone systems and distribution cabling.

Construction:

Copper Conductor

Mica & LSOH Insulation

Al Overall Screen

Vermin-proof & UV Resistant FRPVC Jacket

Conductor: 			    Plain Annealed Solid Copper to IEC 228 Class 1

Fire Barrier:           Mica Tape.

Insulation:  			    LSOH

Twisting:              Two colored insulated conductors are uniformly twisted together to form a pair.

Shape: 			    Circular

Overall Screen:         Overall Polyester/ Aluminum foil screened plus 0.5mm2 copper drain wire

Outer Sheath:		    Vermin-proof & UV Resistant FRPVC

Sheath color:           Black

Temperature Rating:     0 ℃ to 40 ℃

Bending Radius:         6 x overall diameter

Marking:  The cable is marked with white or other colors as follows:

- Caledonian Cables Co., Ltd

- Type and size of cable

- Manufacturing date

- Meter Marking

The meter mark is printed every meter with an accuracy of 0.5%.

Other printing are available according to customer request.

Table 1. Technical Data

| Number of Pair | Nom. Diameter of Conductor (mm) | Nom. Diameter of Cable (mm) |
| --- | --- | --- |
| 1 | 0.8 | 6.5 |
| 2 | 0.8 | 11.0 |
| 4 | 0.8 | 13.5 |

The data included in the present catalogue are merely indicative; Caledonian Cables Limited reserves to itself the right to change them as its own discretion in any time.